# Technical Specification: Open User Experience and Network Path Monitoring Platform

Document purpose: Define a portable, open-source monitoring platform that measures network performance from the end-user perspective across Wi-Fi, switching, Security Gateway security inspection, WAN/ISP, DNS, Core SaaS Productivity Suite, and lateral network paths. The design is intended to meet or exceed the operational visibility previously provided by Aruba UXI while maintaining local ownership of telemetry, long-term trend history, point-in-time evidence, and a verifiable software supply chain.

## 1. Executive Summary

The proposed platform will deploy synthetic monitoring sensors at three representative user locations:

- MDF

- Library

- Classroom wing

Each sensor will connect to the actual production SSID and, where practical, also maintain a dedicated wired management connection. The sensors will continuously emulate common user workflows and measure every observable stage of network access:

1 Wireless RF 2 -> Association 3 -> Authentication 4 -> DHCP 5 -> Default Gateway 6 -> Local/Lateral Services 7 -> Security Gateway 8 -> ISP 9 -> Public DNS 10 -> Internet 11 -> Core SaaS Productivity Suite 12 -> Browser Transaction

Aruba UXI combines physical sensors, software agents, a cloud dashboard, synthetic testing, historical analysis, PCAP capture, onboarding measurements, application testing, and incident grouping. HPE states that UXI sensors act as end users, continuously test network and application services, retain downloadable performance data, and support point-in-time packet captures. [arubanetwo...ng.hpe.com], [hpe.com], [psnow.ext.hpe.com]

The replacement platform shall therefore provide:

- Continuous synthetic user testing

- Wi-Fi association and roaming analysis

- DHCP and DNS timing

- Layer 2 and Layer 3 path testing

- Lateral network testing

- Security Gateway performance correlation

- ISP and Internet path analysis

- Core SaaS Productivity Suite endpoint testing

- Real-browser transactions

- Automated incident evidence collection

- Snapshot and historical trend analysis

- Windows, Linux, macOS, and Android agents

- Open APIs and portable data

- Reproducible configuration

- SBOM generation and vulnerability tracking

# 2. Project Objectives

## 2.1 Primary objective

Determine whether reported slowness following the Legacy Firewall-to-Security Gateway migration originates from:

- Client device

- Wireless RF conditions

- AP association or authentication

- DHCP

- Local switching

- VLAN or Layer 3 routing

- Lateral network services

- Security Gateway inspection

- Security Gateway resource utilization

- WAN circuit

- ISP transit

- DNS

- Core SaaS Productivity Suite

- Application or browser rendering

## 2.2 Secondary objectives

The system shall:

- Establish an evidence-based pre-change and post-change baseline.

- Distinguish perceived slowness from measurable degradation.

- Compare wired and wireless performance from the same location.

- Preserve telemetry for troubleshooting, leadership reporting, and vendor escalation.

- Operate without dependence on a proprietary cloud service.

- Support deployment to additional school sites with minimal customization.

- Produce audit-ready point-in-time diagnostic packages.

- Detect intermittent problems before a user submits a ticket.

## 2.3 Success criteria

The solution will be considered successful when it can answer the following questions within five minutes of an incident:

- Was Wi-Fi signal quality degraded?

- Did the client take too long to associate or authenticate?

- Did DHCP or DNS respond slowly?

- Was the delay present between client and Security Gateway?

- Was Security Gateway inspection or resource usage correlated with the event?

- Did the ISP path exhibit latency, loss, or jitter?

- Was Core SaaS Productivity Suite reachable and responsive?

- Were internal services reachable laterally?

- Was the issue isolated to one sensor, one AP, one VLAN, or the entire site?

- What changed immediately before the degradation?

- Can the condition be reproduced?

- Is there a PCAP, trace, and configuration snapshot for escalation?

# 3. Functional Requirements

## 3.1 Comparison target

The platform shall meet or exceed the following Aruba UXI capabilities:

| Capability | Required replacement behavior |
| --- | --- |
| End-user perspective | Sensors connect through the same production SSID and network path as users |
| Continuous testing | Scheduled tests run continuously, not only during incidents |
| Network onboarding | Measure association, authentication, DHCP, gateway, and DNS stages |
| Application testing | Perform HTTP, HTTPS, API, and browser-based transactions |
| Historical analysis | Retain raw and aggregated time-series data |
| Incident triage | Automatically trigger deeper tests when thresholds are exceeded |
| Packet evidence | Capture bounded PCAPs before or during triggered incidents |
| Trend data | Provide hour, day, week, month, semester, and annual views |
| Roaming visibility | Record BSSID changes, disconnects, reassociations, and roam duration |
| Cross-platform agents | Support Windows, Linux, macOS, and Android |
| Multivendor operation | Remain independent of AP, switch, firewall, and ISP vendors |
| Integrations | Support webhooks, email, Teams, ticketing, and API access |
| Configuration validation | Compare performance before and after network changes |

HPE describes UXI as a vendor-agnostic platform that monitors wired and wireless connectivity, applications, onboarding stages, roaming, and remote user experience using sensors and agents. [hpe.com], [psnow.ext.hpe.com], [developer....tworks.com]

# 4. Proposed Architecture

## 4.1 Logical design

1 +-----------------------------+ 2 ¦ Central Monitoring Platform ¦ 3 ¦-----------------------------¦ 4 ¦ Prometheus or VictoriaMetrics¦ 5 ¦ Grafana ¦ 6 ¦ Loki ¦ 7 ¦ Tempo or Jaeger ¦ 8 ¦ Alertmanager ¦ 9 ¦ Object Storage ¦ 10 ¦ Dependency-Track ¦ 11 +--------------+--------------+ 12 ¦ 13 TLS / VPN / Management VLAN 14 ¦ 15 +---------------------------+---------------------------+ 16 ¦ ¦ ¦ 17 +------v------+ +------v------+ +------v------+ 18 ¦ MDF Sensor ¦ ¦ Library ¦ ¦ Classroom ¦ 19 ¦ Wired + WiFi¦ ¦ WiFi Sensor ¦ ¦ WiFi Sensor¦ 20 +------+------+ +------+------+ +------+------+ 21 ¦ ¦ ¦ 22 +------------- Production SSID/VLAN ------------------+ 23 ¦ 24 AP Layer 25 ¦ 26 Access Switching 27 ¦ 28 Security Gateway LAN / Security 29 ¦ 30 ISP/WAN 31 ¦ 32 DNS / Internet / Core SaaS Application

## 4.2 Architectural principles

### Outside-in testing

Tests shall originate from network locations that represent actual users. Monitoring only Security Gateway, switch, and AP health is insufficient because healthy infrastructure counters do not prove that a user workflow is functioning.

### Dual-vantage monitoring

The MDF sensor should support both:

- Wired production or test VLAN

- Production Wi-Fi SSID

This creates a comparison point:

1 Wi-Fi slow + wired normal = likely wireless issue 2 Wi-Fi slow + wired slow = likely upstream, firewall, ISP, DNS, or application issue

### Centralized analysis with distributed execution

Synthetic tests run locally at each sensor. Metrics, logs, traces, and bounded diagnostic artifacts are sent to a central system.

### Local buffering

Sensors must continue collecting data if the central platform becomes unreachable. Buffered records must be forwarded after connectivity is restored.

### Open telemetry

Telemetry should use open formats wherever practical:

- Prometheus/OpenMetrics for metrics

- OpenTelemetry for traces and structured events

- JSON for event records

- PCAP or PCAPNG for packet evidence

- CycloneDX JSON for SBOMs

- Git-compatible text formats for configuration

Prometheus Blackbox Exporter provides open-source HTTP, HTTPS, DNS, TCP, ICMP, and gRPC probing, making it suitable for the core network and service test engine. [github.com], [oneuptime.com]

# 5. Sensor Specifications

## 5.1 Deployment locations

### MDF sensor

Purpose:

- Establish the site baseline closest to the core and Security Gateway.

- Compare wired and wireless paths.

- Monitor core services and WAN availability.

- Collect SNMP/API information from permitted infrastructure devices.

- Serve as an onsite diagnostic coordinator if the WAN is unavailable.

Recommended interfaces:

- Ethernet management interface

- Wi-Fi test interface

- Optional second Wi-Fi adapter for scanning and passive observations

### Library sensor

Purpose:

- Represent a high-density user area.

- Detect contention, channel utilization, interference, and roaming issues.

- Run web and Core SaaS Productivity Suite transactions through the production SSID.

### Classroom-wing sensor

Purpose:

- Represent typical classroom coverage.

- Identify AP, channel, or building-area-specific problems.

- Detect intermittent performance during instruction hours.

## 5.2 Hardware profile

Minimum sensor requirements:

- Four-core 64-bit processor

- 4 GB RAM minimum, 8 GB preferred

- 32 GB high-endurance storage minimum

- Gigabit Ethernet

- Supported Wi-Fi 6 or later adapter

- Trusted Platform Module where available

- Secure Boot support where practical

- Real-time clock

- Watchdog capability

- UPS or managed PoE power where available

Recommended deployment platforms:

- Raspberry Pi 5 or equivalent ARM64 system

- Intel N100-class mini PC

- Repurposed managed Windows or macOS endpoint

- Android test device for mobile and roaming validation

## 5.3 Wi-Fi adapter requirements

The sensor Wi-Fi adapter must expose, to the extent supported by the OS and driver:

- SSID

- BSSID

- Channel

- Frequency

- Channel width

- RSSI

- Signal quality

- Noise floor, if available

- Transmit rate

- Receive rate

- MCS

- Spatial streams

- Authentication type

- Disconnect reason

- Association event

- Reassociation event

- Roaming event

- Retry or failure counters

On Linux, wpa_supplicant provides a control interface for status information and event notifications and can control authentication, association, and roaming where supported by the driver. [wireless.d...kernel.org], [w1.fi], [w1.fi]

# 6. Layer-by-Layer Test Matrix

## 6.1 Layer 1: Physical and RF

Collect:

- RSSI

- Noise floor

- Signal-to-noise ratio

- Channel

- Channel width

- PHY mode

- Tx/Rx rates

- Link quality

- Interface resets

- Radio driver errors

- Power state

- Sensor CPU temperature

- Ethernet link state and negotiated speed

Threshold examples:

- RSSI warning below -67 dBm

- RSSI critical below -75 dBm

- SNR warning below 25 dB

- SNR critical below 15 dB

- Unexpected rate fallback

- Repeated interface reset

- Wired interface negotiating below expected speed

These values shall be configurable per site and should initially be treated as operational baselines rather than absolute proof of failure.

## 6.2 Layer 2: Wi-Fi and switching

Measure:

- Scan-to-association duration

- Authentication duration

- 802.1X/EAP duration when applicable

- Association success rate

- Disconnect frequency

- BSSID changes

- Roam duration

- VLAN assignment

- ARP resolution time

- Duplicate IP indicators

- Gateway MAC consistency

- Local switch reachability

- Broadcast and multicast behavior where authorized

Required test sequence:

1 Start Wi-Fi connection attempt 2 -> Discover target SSID 3 -> Select BSSID 4 -> Associate 5 -> Authenticate 6 -> Receive VLAN assignment 7 -> Record assigned network 8 -> Resolve gateway MAC 9 -> Verify local reachability

## 6.3 Layer 3: Addressing and routing

Measure:

- DHCP discovery-to-offer time

- DHCP request-to-acknowledgement time

- Assigned address

- Prefix length

- Default gateway

- DNS servers

- Lease duration

- IPv4 and IPv6 status

- Gateway latency

- Packet loss

- Path MTU

- Route changes

- Trace path to defined targets

Tests shall identify:

- Slow DHCP response

- Incorrect DHCP options

- Wrong VLAN

- Wrong gateway

- Overlapping subnet

- Asymmetric path

- IPv6 preferred-path problems

- MTU or fragmentation problems

## 6.4 Layer 4: Transport

Measure TCP connection establishment to:

- Security Gateway-accessible internal service

- District DNS server

- Core SaaS Productivity Suite endpoints

- Public web endpoint

- Other approved SaaS services

Capture:

- TCP connect duration

- Retransmission indicators

- Connection reset

- Timeout

- TLS port availability

- QUIC/UDP reachability where applicable

## 6.5 Layer 5 and 6: Session, TLS, and security inspection

Measure:

- TLS handshake duration

- Certificate chain

- Certificate issuer

- Certificate expiration

- TLS version

- Cipher suite

- Security Gateway inspection certificate presence

- Session reuse

- HTTP/2 or HTTP/3 negotiation

- Unexpected certificate substitution

- Authentication redirects

This layer is particularly important when comparing Security Gateway against Legacy Firewall. A slow TLS handshake or certificate-chain operation may indicate:

- Deep SSL inspection overhead

- Certificate validation delay

- DNS delay

- Session handling delay

- Remote server delay

- Client trust-store or proxy issue

## 6.6 Layer 7: DNS, web, SaaS, and browser experience

Test categories:

### DNS testing

Test both:

- Direct queries to each configured district DNS server

- Queries using the client’s normal OS resolver path

Measure:

- Query duration

- Response code

- Answer correctness

- TCP fallback

- IPv4 record response

- IPv6 record response

- Cache-hit versus cache-miss timing

- DNSSEC outcome where applicable

- Internal and external namespace resolution

Blackbox Exporter supports configurable DNS queries, expected return codes, answer validation, TCP or UDP transport, and DNS over TLS. [github.com], [deepwiki.com]

### HTTP and HTTPS testing

For each target, record:

- DNS duration

- TCP duration

- TLS duration

- Time to first byte

- Content download duration

- Total response duration

- HTTP status

- Redirect count

- Content validation result

- Certificate validity

- Bytes transferred

### Real-browser testing

Use Playwright or an equivalent open-source browser automation framework to execute:

- Public website load

- District homepage load

- Core SaaS Productivity Suite landing page load

- Approved SharePoint page load

- Approved application sign-in landing page

- Static Microsoft CDN object retrieval

- Teams web landing page or non-invasive readiness workflow

Browser tests must collect:

- Navigation timing

- DOM content loaded

- Largest Contentful Paint

- First Contentful Paint

- Total Blocking Time where available

- Resource waterfall

- Failed requests

- DNS, TCP, TLS, and request phases

- Screenshot on failure

- Browser console errors

- HAR file on failure

- Trace file on failure

Simple HTTP tests do not render JavaScript or load all browser resources, while real-browser checks provide a closer representation of what an end user experiences. [oneuptime.com], [thenewstack.io]

# 7. Lateral Network Testing

Lateral testing is required to distinguish Internet slowness from campus or district internal-path problems.

## 7.1 Required lateral paths

From each sensor, test approved targets in the following categories:

- Sensor to sensor

- Sensor to local gateway

- Sensor to local DNS

- Sensor to local DHCP relay or server, where directly testable

- Sensor to local file or print service

- Sensor to district data center

- Sensor to authentication services

- Sensor to internal application

- Sensor to adjacent VLAN test endpoint

- Wired sensor to wireless sensor

- Wireless sensor to wired service

- Site to site across district WAN

## 7.2 Sensor-to-sensor testing

Each sensor shall expose an authenticated test service accessible only from approved management or test networks.

Measure:

- ICMP latency and loss

- TCP connection time

- Bidirectional throughput

- UDP loss and jitter

- Path MTU

- Route path

- Time synchronization offset

Use iperf3 or an equivalent open-source tool for scheduled, rate-limited throughput testing.

Throughput tests must:

- Avoid instructional peak periods unless troubleshooting requires them.

- Use defined bandwidth limits.

- Run in both directions.

- Avoid saturating the WAN.

- Identify the source and destination sensor.

- Record wired versus wireless transport.

- Require authentication or firewall restriction.

## 7.3 Lateral segmentation validation

The platform shall perform explicitly authorized reachability tests to confirm that segmentation remains effective.

Examples:

- Student VLAN cannot reach management interfaces.

- Production clients can reach approved DNS and application services.

- Guest clients cannot reach internal resources.

- Sensor management interfaces are inaccessible from client VLANs.

- East-west access matches documented policy.

This must be implemented as a allowlisted validation process, not as unrestricted network scanning.

# 8. Security Gateway Correlation

Synthetic monitoring alone can identify the affected path, but Security Gateway telemetry is required to determine whether inspection or resource pressure is contributing to the slowdown.

## 8.1 Security Gateway metrics

Collect through SNMP, syslog, API, or FortiAnalyzer where available:

- CPU utilization

- Memory utilization

- Session count

- Session setup rate

- Conservation mode

- Interface utilization

- Interface errors and drops

- WAN latency and packet loss

- SD-WAN health-check results

- DNS filter latency or errors

- Web-filter events

- IPS events

- Antivirus events

- SSL inspection events

- Denied sessions

- Policy identifier

- UTM profile

- HA state, if applicable

- NPU/ASIC offload indicators where available

- FortiGuard reachability

- License and service state

## 8.2 Policy-aware correlation

Where logging permits, each synthetic test should carry a recognizable source identity:

- Reserved sensor IP address

- Sensor hostname

- Dedicated device group

- Optional approved HTTP user-agent marker

- Test identifier in request headers for controlled internal applications

Dashboards shall correlate synthetic test duration with:

- Firewall policy

- Security profile

- CPU and memory

- Session rate

- WAN health

- Interface drops

- Inspection events

## 8.3 A/B test capability

The platform shall support controlled comparison of:

- Wired versus wireless

- Inspected versus minimally inspected policy

- Primary DNS versus secondary DNS

- ISP circuit A versus circuit B

- IPv4 versus IPv6

- HTTP/2 versus HTTP/3 where controllable

- Production SSID versus designated test SSID

Any reduced-inspection test must be change-controlled, narrowly scoped to sensor addresses, time-bounded, and documented.

# 9. Core SaaS Productivity Suite Testing

## 9.1 Test scope

Core SaaS Productivity Suite testing shall include approved, non-destructive checks for:

- DNS resolution

- TCP 443 availability

- TLS negotiation

- Authentication landing-page availability

- Core SaaS Productivity Suite portal access

- SharePoint Online access

- Exchange Online web endpoint availability

- Teams web service reachability

- Microsoft CDN object retrieval

## 9.2 Authentication safety

Tests must not:

- Store user passwords in clear text.

- Repeatedly trigger MFA.

- Generate false risky-sign-in alerts.

- Modify mailbox, Teams, SharePoint, or OneDrive content.

- Use a privileged account.

If authenticated testing is required, use:

- Dedicated synthetic identity

- Least privilege

- Conditional Access documentation

- Protected secrets store

- Certificate-based or workload identity where supported

- Explicit test labels in identity and sign-in logs

## 9.3 Measurements

Capture:

- DNS duration

- TCP connect duration

- TLS duration

- Redirect duration

- Authentication-page duration

- Static asset performance

- Page load timing

- Failed resource requests

- Browser console errors

- HTTP status

- Service-specific success criteria

# 10. Client Roaming Testing

## 10.1 Fixed sensor monitoring

Stationary sensors shall record unexpected changes in:

- BSSID

- Channel

- RSSI

- Association state

- Authentication state

- Default gateway

- Assigned IP

- Connectivity

Unexpected roaming by a stationary device can indicate:

- AP instability

- RF interference

- Load balancing

- Band steering

- Minimum-RSSI enforcement

- Channel or power changes

## 10.2 Mobile roaming agent

True roaming validation requires a moving client. The mobile agent should run on an Android device or managed laptop and record:

- Timestamp

- Location label or test waypoint

- Previous BSSID

- New BSSID

- Channel

- RSSI before roam

- RSSI after roam

- Roam start

- Roam completion

- Packet-loss burst

- Voice or UDP interruption

- DNS result

- Gateway continuity

- Application transaction continuity

Aruba UXI specifically distinguishes fixed sensor testing from deeper roaming and mobile-client analysis, including Android and Zebra agents. [hpe.com], [securewire...sworks.com]

## 10.3 Roaming test workflow

1 Begin continuous ICMP and UDP test 2 -> Start approved voice-quality simulation 3 -> Walk documented route 4 -> Record BSSID transitions 5 -> Record packet-loss burst 6 -> Record application interruption 7 -> Generate route-level report

# 11. Open-Source Tooling Stack

## 11.1 Recommended core stack

| Function | Preferred tool |
| --- | --- |
| Time-series collection | Prometheus |
| Long-term scalable metrics | VictoriaMetrics, optional |
| Dashboarding | Grafana |
| Alerting | Alertmanager |
| ICMP, TCP, DNS, HTTP probes | Prometheus Blackbox Exporter |
| Browser transactions | Playwright |
| Throughput | iperf3 |
| Path analysis | mtr and traceroute |
| Packet capture | tcpdump, dumpcap, or Wireshark CLI |
| Wireless control and events | wpa_supplicant/wpa_cli on Linux |
| Logs | Loki |
| Traces | OpenTelemetry Collector plus Tempo or Jaeger |
| Host metrics | Node Exporter or OS-native collector |
| Container runtime | Podman preferred, Docker acceptable |
| SBOM generation | Syft or cdxgen |
| Vulnerability scanning | Grype and/or Trivy |
| SBOM management | OWASP Dependency-Track |
| Configuration management | Ansible |
| Source/configuration history | Git |

Prometheus Blackbox Exporter supports the core external probes required by this design.  CycloneDX supports SBOM, SaaSBOM, hardware, cryptography, operations, VEX, and additional software supply-chain representations in machine-readable formats. [github.com] [github.com], [cyclonedx.org]

## 11.2 Portability model

### Linux

Linux shall be the reference sensor platform because it provides the deepest Wi-Fi event access and straightforward container deployment.

### Windows

Windows agent support shall include:

- ICMP

- DNS

- TCP

- TLS

- HTTP/HTTPS

- Browser transactions

- Interface statistics

- Wi-Fi connection details available through Windows APIs

- Local buffering

- OpenTelemetry export

### macOS

macOS agent support shall include:

- ICMP

- DNS

- TCP

- TLS

- HTTP/HTTPS

- Browser transactions

- Interface statistics

- Available Wi-Fi telemetry

- Local buffering

- OpenTelemetry export

### Android

Android support shall prioritize:

- SSID and BSSID where OS policy permits

- Signal strength

- Network transitions

- DNS and HTTP checks

- Browser or WebView transaction checks

- Roaming events

- Mobile walkthrough tests

- Local caching and delayed upload

OS privacy controls may limit Wi-Fi fields, particularly BSSID, channel, and scanning behavior. The implementation shall document platform-specific gaps rather than treating absent measurements as successful tests.

# 12. Test Scheduling

## 12.1 Normal cadence

| Test | Frequency |
| --- | --- |
| Sensor health | 15 seconds |
| Gateway ICMP | 10 seconds |
| Internal DNS query | 30 seconds |
| Public DNS query | 30 seconds |
| Security Gateway reachability | 10 seconds |
| ISP next-hop test | 10 seconds |
| Public Internet ICMP | 10 seconds |
| TCP 443 connectivity | 30 seconds |
| Lightweight HTTP check | 60 seconds |
| Core SaaS Productivity Suite HTTPS check | 60 seconds |
| Browser transaction | 5 minutes |
| Lateral service check | 60 seconds |
| Sensor-to-sensor latency | 10 seconds |
| Rate-limited throughput test | 15 to 60 minutes |
| Route trace | 15 minutes or on incident |
| Wireless status collection | 10 seconds |
| Full software inventory | Daily |
| SBOM generation | Per release and daily deployed-state check |
| Configuration snapshot | Daily and before/after change |

## 12.2 Incident cadence

When a threshold violation occurs, the sensor shall temporarily increase diagnostic depth:

1 Trigger condition detected 2 -> Repeat test to eliminate one-sample noise 3 -> Capture Wi-Fi state 4 -> Capture addressing and route state 5 -> Run gateway, DNS, ISP, and application probes 6 -> Execute MTR/path trace 7 -> Correlate Security Gateway metrics 8 -> Start bounded PCAP 9 -> Run browser trace if applicable 10 -> Preserve evidence bundle 11 -> Open or update incident

Automatic escalation must use rate limiting to prevent packet-capture storms or excessive browser activity.

# 13. Point-in-Time Snapshot Requirements

## 13.1 Snapshot contents

Each incident snapshot shall include:

- Timestamp in UTC and local time

- Sensor ID and location

- Sensor version

- OS and kernel version

- Current connected SSID and BSSID

- Channel and frequency

- RSSI and available radio data

- IP configuration

- DHCP server

- DNS servers

- Default gateway

- ARP or neighbor entry for gateway

- Routing table

- Current path traces

- Gateway, ISP, DNS, and SaaS test results

- Relevant Security Gateway telemetry

- Recent synthetic test history

- Bounded PCAP

- Browser HAR and trace when applicable

- Screenshot on browser failure

- Running container or package inventory

- Current configuration commit ID

- Hashes for collected evidence

## 13.2 Evidence integrity

Every snapshot package shall contain a manifest with:

- File name

- File size

- SHA-256 hash

- Creation timestamp

- Sensor identifier

- Test identifier

- Collection reason

- Software version

## 13.3 Retention

Recommended retention:

- Raw high-resolution metrics: 90 days

- One-minute aggregates: 13 months

- Hourly aggregates: 3 years

- Incident PCAPs: 30 to 90 days

- Incident summaries and hashes: 3 years

- Configuration snapshots: Duration of system operation

- SBOM and vulnerability results: Duration of deployed release plus 3 years

Retention must be adjusted to district privacy, legal, and storage policies.

# 14. Trend Analysis and Baselines

## 14.1 Required views

Dashboards shall provide:

- Current status

- Last hour

- Last 24 hours

- Last seven days

- Last 30 days

- Semester-to-date

- Year-over-year comparison

- Before/after change comparison

- Site comparison

- Sensor-location comparison

- Wired-versus-wireless comparison

## 14.2 Baseline dimensions

Baseline data by:

- Sensor

- Site

- SSID

- BSSID

- AP

- Channel

- VLAN

- Gateway

- ISP circuit

- DNS server

- Application

- Time of day

- Day of week

- School schedule period

- Firmware/configuration version

## 14.3 Statistical outputs

Calculate:

- Minimum

- Maximum

- Mean

- Median

- 95th percentile

- 99th percentile

- Standard deviation

- Packet-loss percentage

- Success rate

- Error-budget burn

- Week-over-week change

- Pre-change versus post-change delta

## 14.4 Change markers

Grafana dashboards shall display annotations for:

- Security Gateway cutover

- Policy changes

- Firmware upgrades

- AP changes

- Switch changes

- DNS changes

- ISP maintenance

- Certificate renewal

- School calendar events

- Reported user incidents

# 15. Service-Level Indicators

## 15.1 Core SLIs

### Wi-Fi onboarding

1 Wi-Fi onboarding time = 2 scan + association + authentication + DHCP + gateway validation

### DNS availability

1 Successful valid DNS responses / total DNS attempts

### Application availability

1 Successful validated transactions / total transactions

### Network latency

Track separately:

- Sensor to gateway

- Sensor to ISP

- Sensor to public target

- Sensor to internal target

- Sensor to Core SaaS Productivity Suite

### Roaming interruption

1 Time from last successful packet on old BSSID 2 to first successful packet on new BSSID

## 15.2 Initial performance objectives

Initial objectives should be validated against the existing environment:

- Gateway packet loss: < 0.1%

- Internet packet loss: < 1%

- Internal DNS median: < 50 ms

- Public DNS median: < 100 ms

- TCP setup to common Internet services: < 250 ms

- Local gateway median: < 10 ms over Wi-Fi

- Wi-Fi onboarding: < 5 seconds for PSK and < 10 seconds for 802.1X

- Browser transaction success: >= 99%

- Roam interruption: < 250 ms for voice-sensitive environments

- Synthetic test availability: >= 99.5%

These are starting engineering targets, not guaranteed vendor SLAs.

# 16. Root-Cause Classification

The platform shall assign evidence-based probable cause classifications.

| Observation | Probable domain |
| --- | --- |
| Poor RSSI/SNR and slow gateway response | Wi-Fi RF |
| Good RF but slow association | AP, authentication, or client |
| Fast association but slow DHCP | DHCP server, relay, VLAN, or Security Gateway |
| Fast gateway but slow DNS | DNS service or DNS filtering |
| Wired normal and wireless slow | Wi-Fi/AP/client |
| Wired and wireless slow to gateway | switching, VLAN, routing, or Security Gateway LAN path |
| Gateway normal but ISP hop slow | WAN handoff or ISP |
| Internet normal but Core SaaS Productivity Suite slow | Microsoft path, CDN, TLS, or application |
| TLS slow across many sites | inspection, certificates, DNS, or firewall load |
| One sensor affected | local AP, radio, cable, or client condition |
| All sensors affected simultaneously | core, Security Gateway, ISP, DNS, or shared SaaS |
| Lateral paths slow but Internet normal | switching, routing, internal service, or segmentation |

The system must label these as probable causes, not definitive causes, unless supported by direct telemetry.

# 17. Alerting and Incident Management

## 17.1 Alert levels

### Informational

- BSSID change

- Configuration revision

- Software update

- Baseline deviation without service impact

### Warning

- Elevated DNS response time

- Increased packet loss

- Slow browser transaction

- RSSI below operational target

- Single-sensor degradation

### Critical

- Failed onboarding

- Default gateway unavailable

- Multiple sensors affected

- Core SaaS Productivity Suite transaction failure

- Persistent ISP loss

- Security Gateway conservation mode

- DNS outage

- Significant roaming interruption

## 17.2 Correlation requirements

Similar alerts shall be grouped by:

- Time window

- Site

- Shared target

- Shared AP/BSSID

- VLAN

- ISP circuit

- DNS server

- Application

- Probable fault domain

This emulates the incident-grouping behavior that HPE describes as part of UXI’s AIOps workflow. [hpe.com], [psnow.ext.hpe.com]

## 17.3 Ticketing integration

The alert payload shall contain:

- Incident ID

- Site

- Sensor locations affected

- Start time

- Current status

- Failed layers

- Probable fault domain

- Dashboard link

- Evidence-package link

- Relevant change marker

- Recommended next diagnostic action

# 18. Security and Privacy

## 18.1 Network security

- Sensors must use dedicated management identities.

- Management traffic must be encrypted.

- Administrative interfaces must not be exposed to production client networks.

- Central ingestion endpoints must require mutual TLS or equivalent authentication.

- Sensor-to-sensor testing must be allowlisted.

- Throughput services must be firewall-restricted.

- Secrets must be stored outside source-controlled configuration.

- Sensors must use least privilege.

- Container capabilities must be minimized.

## 18.2 Packet capture controls

Packet capture shall:

- Be disabled by default except for bounded incident triggers.

- Exclude or redact sensitive payloads where practical.

- Use capture filters.

- Limit duration and size.

- Encrypt artifacts at rest.

- Restrict access.

- Log every access and download.

- Follow district retention policy.

## 18.3 Student and staff privacy

The system shall not collect:

- Browsing history from actual users

- Application content

- User credentials

- Message content

- Student identifiers

- Full unfiltered packet captures

Synthetic tests must use controlled test identities and predefined destinations.

# 19. SBOM and Software Supply-Chain Requirements

## 19.1 Required SBOM format

Every sensor and central-platform release shall generate:

- CycloneDX JSON SBOM

- Optional SPDX SBOM

- Container-image digest

- Package inventory

- License inventory

- Build provenance

- Git commit

- Release manifest

CycloneDX is an OWASP and ECMA-424 standard covering multiple bill-of-materials types and supporting machine-readable JSON, XML, and Protocol Buffers. [github.com], [cyclonedx.org]

## 19.2 SBOM lifecycle

1 Source commit 2 -> Build 3 -> Generate SBOM 4 -> Scan dependencies 5 -> Sign artifact 6 -> Publish image 7 -> Deploy pinned digest 8 -> Upload SBOM to Dependency-Track 9 -> Continuously assess vulnerabilities

## 19.3 Required controls

- No production use of unpinned latest container tags.

- Container images must be referenced by version and digest.

- SBOM must be generated at build time.

- Deployed-state inventory must be checked daily.

- Critical vulnerabilities must trigger review.

- Exceptions must include owner, justification, and expiration.

- Updates must be tested in staging.

- Rollback artifacts must be retained.

# 20. Deployment and Configuration Management

## 20.1 Deployment model

Preferred:

- Containers for portable test and exporter services

- Ansible for Linux sensor provisioning

- PowerShell for Windows deployment

- Signed package or MDM deployment for macOS

- Managed APK or MDM deployment for Android

- Git repository for configuration

- Environment-specific inventory files

## 20.2 Configuration hierarchy

1 global defaults 2 -> district defaults 3 -> site defaults 4 -> sensor overrides

## 20.3 Required configuration fields

- Sensor ID

- Site

- Physical location

- Management endpoint

- Production SSID profile reference

- Approved internal targets

- Approved external targets

- Test intervals

- Thresholds

- Capture policy

- Retention policy

- Maintenance windows

- Notification routes

## 20.4 Secret handling

Secrets shall not appear in:

- Git commits

- Dashboard variables

- Alert payloads

- PCAP manifests

- SBOMs

- Unencrypted local configuration

Use an approved secrets manager, protected OS credential store, or encrypted Ansible vault.

# 21. Acceptance Test Plan

## Phase 1: Platform validation

- All sensors register successfully.

- Central platform receives metrics, logs, and traces.

- Dashboards identify each sensor and location.

- Local buffering survives a central-platform outage.

- Alerts reach the designated channel.

- SBOM is generated and ingested.

## Phase 2: Network onboarding

For every sensor:

- Join production SSID.

- Record association timing.

- Record authentication timing.

- Obtain DHCP lease.

- Validate gateway.

- Validate internal DNS.

- Validate public DNS.

- Validate Internet.

- Validate Core SaaS Productivity Suite.

- Complete browser transaction.

## Phase 3: Fault injection

Safely simulate:

- DNS failure

- Increased latency

- Packet loss

- Blocked TCP 443

- Incorrect DNS response

- Sensor central-uplink loss

- Browser transaction failure

- BSSID change

- Security Gateway policy change marker

Expected result:

- Correct alert

- Correct probable fault domain

- Snapshot generation

- Dashboard annotation

- Recovery detection

## Phase 4: Security Gateway comparison

Measure under identical conditions:

- Wired versus Wi-Fi

- Security inspection enabled versus approved sensor-only reduced inspection

- Primary versus secondary DNS

- Multiple time periods

- Normal versus peak utilization

## Phase 5: Seven-day pilot

Success requirements:

- At least 99% sensor telemetry availability

- No uncontrolled network saturation

- No user-impacting test traffic

- Complete trend dashboards

- Successful alert grouping

- At least one usable point-in-time evidence package

- Documented comparison between perceived and measured performance

# 22. Deliverables

- Architecture diagram

- Sensor build standard

- Ansible and PowerShell deployment methods

- Central platform deployment files

- Sensor inventory

- Test-target inventory

- Grafana dashboards

- Alert rules

- Security Gateway correlation dashboard

- Core SaaS Productivity Suite transaction suite

- Roaming test procedure

- Snapshot evidence procedure

- Data-retention standard

- Security and privacy assessment

- CycloneDX SBOM

- Vulnerability report

- Operations runbook

- Pilot findings report

- Before-and-after Security Gateway performance report

- Production acceptance record

# 23. Implementation Recommendation

## Phase 1: Minimum viable deployment

Deploy:

- Prometheus

- Grafana

- Alertmanager

- Blackbox Exporter

- Playwright

- Loki

- Three Linux-based sensors

- Security Gateway SNMP/syslog ingestion

Start with:

- Gateway

- DNS

- ISP

- Public Internet

- Core SaaS Productivity Suite

- One internal service

- Sensor-to-sensor latency

- Browser transaction

## Phase 2: UXI-equivalent diagnostics

Add:

- Wireless event exporter

- Association and DHCP timing

- Incident-triggered PCAP

- HAR and browser traces

- Security Gateway policy correlation

- Configuration annotations

- Lateral throughput testing

- Automated incident evidence packages

## Phase 3: UXI-plus capabilities

Add:

- Windows agent

- macOS agent

- Android roaming agent

- OpenTelemetry traces

- SBOM pipeline

- Dependency-Track

- Statistical baselines

- Anomaly detection

- Cross-site comparison

- Service-level reporting

# 24. Recommended Final Platform

1 Sensors: 2 Linux reference sensors at MDF, Library, Classroom Wing 3 Windows/macOS agents for endpoint validation 4 Android device for roaming walkthroughs 5 6 Testing: 7 Blackbox Exporter 8 Playwright 9 iperf3 10 mtr 11 wpa_supplicant event collection 12 bounded tcpdump/dumpcap 13 14 Central: 15 Prometheus or VictoriaMetrics 16 Grafana 17 Alertmanager 18 Loki 19 OpenTelemetry Collector 20 Tempo 21 S3-compatible object storage 22 23 Security: 24 mTLS 25 protected secrets 26 Git-based configuration 27 signed builds 28 pinned container digests 29 30 Supply chain: 31 CycloneDX 32 Syft/cdxgen 33 Grype/Trivy 34 Dependency-Track

This design will not duplicate Aruba UXI through one application. Instead, it creates an open, modular user-experience platform that offers deeper path separation, longer retention, lateral testing, Security Gateway correlation, portable data, reproducible configurations, and verifiable software components.

## Writing and specification approach

The specification was structured around measurable requirements, operational evidence, and acceptance criteria rather than a simple list of monitoring products. It separates each network layer, defines the data needed to isolate Security Gateway performance, incorporates cross-platform constraints, and adds SBOM, trend, snapshot, privacy, and change-correlation requirements that exceed a basic synthetic-monitoring deployment.